Allow PS/2 mouse movements when SWAT is active.

This problem, which has plagued us from many months, occurs on MC

systems with a mouse attached through the mouse port (but not COM1 or

COM2).	If SWAT is active and the mouse is moved, the system locks up.

The nub of the problem is that in the above environment, the mouse

communicates to the system through the 8042 PPI.  In particular, the

sequence of events is as follows:

1.  The mouse is moved (usually inadvertently such as by hitting the

table or keyboard hard enough to jar it).

2.  A byte from the mouse is placed in the PPI output port buffer and

latched there meaning that no other byte can be put there until

this byte has been cleared.

3.  If the mouse interrupt has been enabled (such as by a mouse

driver), IRQ2 (the cascade interrupt) is signalled but remains

pending because it is masked off.

4.  If the mouse interrupt has been enabled (such as by a mouse

driver), IRQ12 (the mouse interrupt) is signalled but remains

pending because it is masked off.

5.  A keystroke occurs but is ignored because there is already a

latched byte in the PPI output port buffer.  At this point the

keyboard is dead.  SWAT is in a tight loop waiting for a keyboard

interrupt (IRQ1) but none occurs because the mousestroke hasn't

been cleared from the PPI.

To fix this problem, we install two new interrupt handlers whenever

SWAT becomes active:  one for each of IRQ2 and IRQ12.  In the process,

several other changes are made.

* The SAVE_KEYB/SAVE_TIMER routines are combined into one (SAVE_IRQS)

and the IRQ2 and IRQ12 interrupts are handled there as well.

* Similarly, the REST_KEYB/REST_TIMER routines are combined into

REST_IRQS and the IRQ2 and IRQ12 interrupts are restored there.

* The ENABLE_KBTM routine is renamed to ENABLE_IRQS and the IRQ2 and

IRQ12 interrupts are enabled there as well.

* IRQ2 and IRQ12 interrupt handlers are defined to clear the

mousestroke so the keyboard can continue to function.

* A common routine to drain the PIQ is defined and used widely.  This

routine also attempts to deal with the sticky problem of 486 cache

obviating the JMP SHORT $+2 delays if the prefetch is satisfied from

the cache.  If we're on a Micro Channel system, we write a byte to

I/O port 4Fh, a technique used by IBM in its 486 BIOSes.  The idea

is that this I/O port (whose meaning is undefined) will (by virtue

of the I/O activity), cause a sufficient delay to overcome

back-to-back I/O problems.  As you can see, I originally thought

that this might be the problem and did all the work to no avail.

Nonetheless, this is a good idea, so I left it in.

* Fix a bug in the ENABLE_NMI routine on MC systems where we don't

clear enough bits in the arbitration register.  I know of no problem

caused by this, but it can't hurt to follow the spec.

The modules changed and why are as follows:

SWAT.ASM	 Coalesce and rename SAVE_xxx, REST_xxx, ENABLE_KBTM

routines.

SWAT_CMD.ASM	 Define IMR command to read and display the master and

slave PIC IMRs.

SWAT_DTE.ASM	 When displaying the IDT (via F4), temporarily restore

the original IDT entries for IRQ1 and IRQ12 (as we

already do for IRQ0 and IRQ1).

SWAT_FVM.ASM	 Call DRAINPIQ as needed.

SWAT_INI.ASM	 Call DRAINPIQ as needed.

SWAT_INT.ASM	 Call DRAINPIQ as needed.

SWAT_IOP.ASM	 Define IMR command to read and display the master and

slave PIC IMRs.

Read in and display both master and slave IMR, IRR,

and ISR bytes.

Call DRAINPIQ as needed.

SWAT_KEY.ASM	 Define IRQ2 and IRQ12 handlers.

Coalesce and rename SAVE_xxx, REST_xxx, ENABLE_KBTM

routines.  Handle IRQ2 and IRQ12 therein.

Define DRAINPIQ routine and call as needed.

SWAT_VCP.ASM	 Call DRAINPIQ as needed.

When enabling NMI, clear all bits but Bit 7 in the

arbitration register to enable disk activity.

SWAT_VID.ASM	 Call DRAINPIQ as needed.

8253.INC	 Define @8253_XCIO as 4Fh

8255.INC	 Define @S2C_AUXDIS and @S2C_AUXENA PPI commands.