Implement INSERT Command

To debug Windows at the lowest level, we need to be able to insert

ourselves into Windows startup shortly after it enters PM.  To this

end, the INSERT command is available.  It is used from the SWAT

command line at the point just before Windows enters PM.  The syntax

of the command is as follows:

INSERT gdtr idtr cr3 lapde [tr ldtr]

INSERT *    idtr

where gdtr = linear address of fword describing the GDT

idtr = ...				    IDT

cr3  = incoming CR3 (optional)

lapde= linear address of 4MB block where SWAT PDE(s) should be copied

tr   = task register

ldtr = LDT register

At the moment, the TR and LDTR fields are not used.

To use this command, find the point at which Windows is just about to

enter PM.  At startup, there's a message which reads

....The Windows callback address is xxxx:xxxx.

Then follow these steps:

1.  Bring up SWAT and set a breakpoint on the Windows callback

address.

2.  Start Windows.

3.  After a short pause as Windows loads various RM drivers, the above

breakpoint trips.

4.  Type U .RETF to disassemble instructions at the return address

which is in Windows code.

5.  Page down a few screens until you get to code which enters PM by

setting the the PE bit in CR0.

6.  Along the way, you should also note from where the new values for

CR3, GDTR, and IDTR are loaded.  For example, in the initial release

of Win98, CR3 comes from DS:000D, GDTR from DS:0480, and IDTR from

DS:0486.

7.  Type on the SWAT command line

insert ds:480 ds:486 {ds:d ff000000

This command line is interpreted as the fword which contains the GDTR

is at ds:480, the fword for the IDTR is at ds:486, just beyond the

GDTR, the incoming CR3 is the dword at ds:d (the left brace extracts a

dword from the address to its right), and SWAT should use the linear

address FF000000 (must be multiple of 4MB) for its code.  SWAT uses

TSSs for its interrupts, grabbing entries from the end of the GDT.

8.  Find an instruction after Windows enters PM at which SWAT can

safely stop.  This means that at least SS:ESP has been specified, and

preferably the othe segment registers as well.

9.  Place a breakpoint on this instruction (I usually just put this

instruction at the top of the screen and press PadStar).

10.  If it all works, SWAT should come up in PM under Windows!

11.  Of course, it can't be all that easy.  At this point you've

successfully intruded into the Windows startup code's GDT and IDT, but

there's another IDT into which SWAT needs to intrude.  Continue

single-stepping until the first CALL occurs, and step into this call.

You'll notice a LOOPD followed by an LIDT instruction.  Step over the

LOOPD and to the LIDT.	At the LIDT instruction, type on the SWAT

command line

insert * .ea

which tells SWAT to intrude into this IDT as well.  That is, the

asterisk is a placeholder for the GDTR (into which we've already

intruded), and the new IDTR is the effective address of the data

referenced by the current instruction (LIDT), which is what .ea

returns.

12.  From this point on you can wade through Windows startup code

including some VxDs.  The point at which you can have trouble is VPICD

which rebases the Master PIC (Programmable Interrupt Controller), from

08h to 50h.